The Exact INMP441 to ESP32 Wiring Guide

The INMP441 microphones are strictly 3.3V devices. Do not connect them to 5V, or they will be permanently damaged. The ESP32's 3.3V pin can safely power all three (they only draw ~1.5mA each).

Here is the exact pin mapping you need to solder or breadboard when the parts arrive:

Power & Ground (All 3 Mics):

- VDD $\rightarrow$ ESP32 3.3V

- GND $\rightarrow$ ESP32 GND

Mic 1 (Front Left) - assigned to I2S0, Left Channel:

- L/R Pin $\rightarrow$ GND (This sets it to the Left channel)

- SCK (Serial Clock) $\rightarrow$ ESP32 Pin 14

- WS (Word Select/LRC) $\rightarrow$ ESP32 Pin 15

- SD (Serial Data) $\rightarrow$ ESP32 Pin 32

Mic 2 (Front Right) - assigned to I2S0, Right Channel:

- L/R Pin $\rightarrow$ 3.3V (This sets it to the Right channel)

- SCK $\rightarrow$ ESP32 Pin 14 (Shared with Mic 1)

- WS $\rightarrow$ ESP32 Pin 15 (Shared with Mic 1)

- SD $\rightarrow$ ESP32 Pin 32 (Shared with Mic 1)

Mic 3 (Rear Center) - assigned to I2S1, Left Channel:

- L/R Pin $\rightarrow$ GND (Sets it to Left channel)

- SCK $\rightarrow$ ESP32 Pin 26

- WS $\rightarrow$ ESP32 Pin 27

- SD $\rightarrow$ ESP32 Pin 33

Here is the exact distinction to clear that up before we write the code:

- The Mics (INMP441) are Components: They are delicate logic chips that strictly operate at 3.3V. If you touch 5V to them, they will burn out instantly.

- The ESP32-CAM is a Development Board: The main silver ESP32 chip on that board also runs at 3.3V. However, the board itself has a built-in voltage regulator. When the WiFi radio and the OV3660 camera turn on at the exact same millisecond, they pull a massive surge of current.

  - If you try to power the board via its 3.3V pin, you bypass that regulator. Most external 3.3V sources cannot handle that sudden amp draw, the voltage drops, and the board crashes (brownout).

  - By feeding 5V into the board's 5V pin, you allow the onboard regulator to absorb that current spike and smoothly deliver a steady 3.3V to the camera and WiFi chip.

The Golden Wiring Rule for Vanguard:

- 12V Metek Box $\rightarrow$ Buck Converter (tuned to 5V) $\rightarrow$ ESP32-CAM 5V Pin.

- ESP32-CAM 3.3V Pin $\rightarrow$ INMP441 Mics VDD. (The ESP32-CAM generates a clean 3.3V that is safe to share with the low-power mics).

This is the final bridge to make Vanguard physically move using your new Tri-Node architecture. We are going to establish a wireless link from your laptop to the ESP-WROOM-32, and then a hardwired Serial link from the ESP-WROOM-32 down to the Arduino Uno.

Here is exactly how to wire the microcontrollers together and the code required for each step of the chain.

1. The Wiring: ESP-WROOM-32 to Arduino Uno

We are going to use UART Serial Communication. The ESP-WROOM-32 has three hardware serial ports. We will use Serial2 (Pins 16 and 17) so we keep the main USB serial port free for debugging.

On the Arduino Uno, we will use SoftwareSerial on Pins 12 and 13. This keeps the Uno's main TX/RX pins free so you can still upload code via USB without having to unplug wires every time.

The Connections:

- Ground (CRITICAL): ESP-WROOM-32 GND $\rightarrow$ Arduino Uno GND. (Without a shared ground, the signals will float and the motors will twitch randomly).

- Command Line: ESP-WROOM-32 TX2 (GPIO 17) $\rightarrow$ Arduino Uno Pin 12 (RX).

⚠️ The 5V Logic Trap: The Arduino Uno operates at 5V, while the ESP32 operates at 3.3V. The Uno will safely read the 3.3V signal from the ESP32's TX pin. However, do not connect the Uno's TX pin back to the ESP32's RX pin directly. A 5V signal from the Uno will fry the ESP32's RX pin. For driving Vanguard, we only need one-way communication (ESP $\rightarrow$ Uno), so we leave the Uno's TX pin completely disconnected.

This is where Vanguard transitions from a remote-controlled car into a true robotics platform. You already know the surface-level concepts of SLAM (Simultaneous Localization and Mapping), so you know that mapping an environment requires highly accurate distance data.

Since we are mounting these sensors rigidly to the front of the chassis, we are doing 1D Forward Occupancy Mapping.

Here is how we fuse the narrow, laser-accurate Time of Flight (ToF) sensor with the wide, noisy Ultrasonic sensor, broken down into our three phases.

Phase 1: Hardware Architecture & Wiring (Node 2)

Both of these sensors will connect to your new ESP-WROOM-32 (Node 2), which is already handling the audio telemetry.

1. The ToF Sensor (Usually VL53L0X or VL53L1X)

ToF sensors use the I2C protocol. They are strictly 3.3V devices and are incredibly easy to wire to the ESP32.

- VCC $\rightarrow$ ESP32 3.3V

- GND $\rightarrow$ ESP32 GND

- SDA (Data) $\rightarrow$ ESP32 Pin 21

- SCL (Clock) $\rightarrow$ ESP32 Pin 22

2. The Ultrasonic Sensor (HC-SR04)

Here is a massive hardware trap that burns out ESP32 boards: Standard HC-SR04 sensors require 5V to operate, but they also output a 5V signal on their Echo pin. The ESP32 pins operate at 3.3V. If you plug the 5V Echo pin directly into the ESP32, you will eventually fry that pin.

You must use a simple voltage divider (using two resistors, like 1kΩ and 2kΩ) on the Echo line to step the 5V down to ~3.3V.

- VCC $\rightarrow$ 5V Buck Converter Output (Do not use the ESP32's 3.3V pin)

- GND $\rightarrow$ ESP32 GND

- TRIG (Trigger) $\rightarrow$ ESP32 Pin 18 (Outputting 3.3V from the ESP32 to trigger the 5V sensor works perfectly fine).

- ECHO (Receive) $\rightarrow$ Voltage Divider $\rightarrow$ ESP32 Pin 19

Let's dive into the robotic arm subsystem. You are building a 2-DOF (Degree of Freedom) Planar Arm with a 1-DOF End Effector. Your analogy of a human arm is perfect:

- Shoulder Joint: Controls the angle of the main boom relative to the base.

- Elbow Joint: Controls the forearm angle relative to the main boom.

- End Effector (Claw): The gripping mechanism at the tip.

Just a quick hardware correction: you mentioned "SG60" servos, but you are likely using the standard blue SG90 9-gram micro servos. If they are SG90s, the following power architecture is critical.

Here is Phase 1 of the Robotic Arm implementation.

Phase 1: Kinematics, Power, and PCA9685 Wiring

Because this arm is lifting its own weight plus whatever it grabs, the servos will experience high torque loads. An SG90 under load can draw up to 700mA. Three of them stalling simultaneously will pull over 2 Amps. If you plug them directly into the Arduino Uno, the Uno will instantly reset or fry its internal voltage regulator.

This is exactly why your decision to use the PCA9685 16-Channel Servo Driver is the right move.

1. The Power Architecture (Life or Death for the Servos)

We need to split the logic power (the data signals) from the motor power (the muscle).

- Logic Power: The Arduino Uno will power the PCA9685's microchip.

- Motor Power: Your 12V 10A Metek box will power the servos, but it MUST go through your Buck Converter first.

- Buck Converter Tuning: Before plugging any servos in, hook your multimeter up to the buck converter and tune the little brass screw until the output is exactly 5.5V or 6.0V. Any higher, and the SG90s will burn out.

2. The PCA9685 to Arduino Uno Wiring

The PCA9685 uses the I2C protocol, which is brilliant because it only requires two pins on your Uno, leaving plenty of room for your motor driver pins.

- Logic Wiring (To the Uno side pins):

  - VCC $\rightarrow$ Arduino Uno 5V

  - GND $\rightarrow$ Arduino Uno GND

  - SDA (Data) $\rightarrow$ Arduino Uno Pin A4

  - SCL (Clock) $\rightarrow$ Arduino Uno Pin A5

- Motor Power Wiring (To the green terminal block):

  - V+ $\rightarrow$ Buck Converter Positive Output (5.5V)

  - GND $\rightarrow$ Buck Converter Negative Output

  - Crucial Step: Ensure there is a common ground wire connecting the Buck Converter negative output to the Arduino Uno GND.

3. The Servo Channel Mapping

Plug your three SG90 servos into the yellow, red, and brown header pins on the PCA9685. The standard color code is Yellow=PWM, Red=V+, Brown=GND.

- Channel 0: Shoulder Joint Servo

- Channel 1: Elbow Joint Servo

- Channel 2: Claw Servo

Since you updated the Python GUI earlier to use a single synchronized slider for the flippers, the logic on the Arduino side needs to catch that single command and distribute it to both servos perfectly.

However, because you are using MG995 servos for the flippers, the mechanical and electrical rules change dramatically compared to the small SG90s.

Phase 3: The MG995 Power Reality Check

The MG995 is a heavy-duty, metal-geared servo. While an SG90 pulls about 700mA when stalled, a single MG995 can pull 1.2 to 1.5 Amps under heavy load. If Vanguard gets its flippers stuck under a piece of rubble and both servos stall, they will yank 3 Amps of current instantly.

- The Buck Converter Requirement: Standard LM2596 buck converters max out around 2.5A to 3A and get incredibly hot. To safely run two MG995s and three SG90s simultaneously from your 12V 10A Metek box, your buck converter should ideally be a 5A model (like the XL4015). If you use a standard 3A converter, stick a small heatsink on it, or the thermal protection will shut it down right when Vanguard starts climbing.

- Wiring: Plug the Left Flipper into Channel 3 and the Right Flipper into Channel 4 on your PCA9685 driver.